Artificial Pancreas Prototype

Wiring & Assembly Guide

Moorthy & Gang  |  Dexathon 2026  |  Team ID DEX26319

SAFETY: Bench prototype only. Use coloured water, never real insulin, and never connect any part to a person. This is not a medical device.

# 1. System overview

The ESP32 reads a glucose signal through an ADS1115 ADC, predicts glucose 30 minutes ahead, and drives two pumps: a stepper-driven syringe pump for insulin (endocrine function) and a peristaltic pump for digestive-enzyme fluid (exocrine function). A web page talks to the ESP32 over Wi-Fi.

# 2. Parts list

| Part | Specification | Purpose |
| --- | --- | --- |
| ESP32 DevKit v1 | 30-pin, 3.3 V logic | Controller and Wi-Fi web server |
| ADS1115 module | 16-bit I2C ADC | Accurate reading of the sensor voltage |
| Glucose electrode strip | Glucose-oxidase screen-printed electrode | Bench glucose sensing |
| Op-amp / potentiostat | MCP6002 or LMP91000 module | Bias and current-to-voltage conversion |
| NEMA 17 stepper | 1.5 A or less, 12 V | Moves the syringe plunger |
| A4988 driver | Stepper driver module | Drives the stepper from ESP32 pins |
| Lead-screw syringe holder | 3D-printed or kit | Converts rotation to plunger travel |
| Peristaltic pump | 12 V DC mini pump | Enzyme fluid delivery |
| IRLZ44N MOSFET | Logic-level N-channel | Switches the peristaltic pump |
| 1N4007 diode | Flyback | Protects MOSFET from pump spikes |
| Push button | Normally open (red) | Emergency stop |
| Active buzzer | 3.3 V - 5 V | Alarms |
| Power | 12 V 2 A adapter + buck converter to 5 V | Motors and electronics |
| Misc. | 100 uF / 25 V capacitor, 220 ohm, 10 k ohm, breadboard, jumpers | Support components |

# 3. ESP32 pin map

| ESP32 pin | Connects to | Notes |
| --- | --- | --- |
| GPIO 26 | A4988 STEP | Step pulses |
| GPIO 27 | A4988 DIR | Direction |
| GPIO 25 | A4988 EN | Active LOW (HIGH = driver off) |
| GPIO 14 | MOSFET gate (via 220 ohm) | Enzyme pump ON/OFF |
| GPIO 21 (SDA) | ADS1115 SDA | I2C data |
| GPIO 22 (SCL) | ADS1115 SCL | I2C clock |
| GPIO 33 | E-stop button to GND | INPUT_PULLUP, LOW = stop |
| GPIO 32 | Buzzer + | Buzzer - to GND |
| GPIO 2 | On-board LED | Status blink |
| 3V3 | ADS1115 VDD, A4988 VDD | Logic supply |
| GND | Common ground | ALL grounds must be joined |

# 4. Wiring steps

## 4.1 Power

- Connect the 12 V adapter to a rail on the breadboard (+12 V and GND).

- Feed a buck converter from the 12 V rail and set it to 5 V; connect its output to the ESP32 VIN and GND.

- Join the 12 V ground, buck ground and ESP32 GND into one common ground.

- Place the 100 uF capacitor across A4988 VMOT and GND, close to the driver.

## 4.2 Insulin pump: A4988 and stepper

| A4988 pin | Connection |
| --- | --- |
| VMOT / GND | +12 V and GND (with 100 uF capacitor) |
| VDD / GND | ESP32 3V3 and GND |
| STEP, DIR, EN | GPIO 26, 27, 25 |
| RESET and SLEEP | Jumper them together |
| MS1, MS2, MS3 | Leave open for full step, or tie high for microstepping (adjust STEPS_PER_UNIT) |
| 1A, 1B, 2A, 2B | Stepper coil A and coil B (find the pairs with a multimeter: about 2 ohm) |

Set the current limit with the small potentiometer before connecting the motor for long runs, following your module's datasheet. Never disconnect the motor while powered.

## 4.3 Enzyme pump: MOSFET switch

- Connect GPIO 14 to the MOSFET gate through a 220 ohm resistor.

- Add a 10 k ohm resistor from gate to GND so the pump stays off during boot.

- Connect the MOSFET source to GND and the drain to the pump negative wire.

- Connect the pump positive wire to +12 V.

- Place the 1N4007 diode across the pump terminals: cathode (striped end) to +12 V, anode to the drain.

## 4.4 ADS1115 (sensor input)

| ADS1115 pin | Connection |
| --- | --- |
| VDD | ESP32 3V3 |
| GND | GND |
| SCL, SDA | GPIO 22, GPIO 21 |
| ADDR | GND (address 0x48) |
| A0 | Output of the sensor front end (section 5) |

## 4.5 Emergency stop and buzzer

- E-stop: one button terminal to GPIO 33, the other to GND. The firmware latches the stop until the board is reset.

- Buzzer: positive to GPIO 32, negative to GND. For a buzzer drawing more than 20 mA use a BC547 transistor with a 1 k ohm base resistor.

# 5. Sensor front end (real-sensor mode)

The glucose strip produces a tiny current (nanoamps) proportional to glucose. A transimpedance stage converts this current to a voltage the ADS1115 can read.

- Apply the working-electrode bias recommended on your strip's datasheet (often 0.2 to 0.6 V versus the reference electrode).

- Use an op-amp (or an LMP91000 module) as a transimpedance amplifier with feedback resistor Rf of 100 k to 1 M ohm: Vout = Vbias + I x Rf.

- Connect Vout to ADS1115 A0 and keep the wires short and shielded. Add a 100 nF capacitor across Rf to reduce noise.

- Allow 60 seconds for the strip to stabilise after a sample is applied.

If you do not have the sensor circuit ready, keep SIM_MODE true in the firmware. A potentiometer on A0 (3.3 V - GND) is also a good stand-in for demos.

# 6. Calibration

## 6.1 Glucose sensor (2 points)

- Prepare glucose solutions of known concentration in PBS buffer, for example 100 mg/dL and 300 mg/dL.

- Record the ADS1115 voltage for each solution (printed in Serial Monitor).

- Compute SLOPE = (300 - 100) / (V300 - V100) and OFFSET = 100 - SLOPE x V100.

- Enter the values in CAL_SLOPE and CAL_OFFSET and re-upload.

## 6.2 Insulin syringe pump

- Fill the syringe with coloured water and mark the starting level.

- Command a known number of steps and measure the volume moved.

- Set STEPS_PER_UNIT so that one 'unit' equals the volume you choose to represent 1 U.

## 6.3 Enzyme pump

- Run the pump for 10 seconds into a measuring syringe.

- Divide the volume (ml) by 10 and enter it as ENZ_ML_PER_SEC.

# 7. First power-up checklist

- Double-check that the MOSFET and A4988 are oriented correctly and that all grounds are common.

- Power from USB first (no motors) and confirm the Serial Monitor prints the IP address.

- Press the e-stop and confirm both pumps stop and the LED blinks quickly.

- Switch on 12 V and run a small simulated meal from the web page; watch the syringe move.

- Confirm the low-glucose suspend triggers by lowering the simulated or potentiometer value.

# 8. Troubleshooting

| Problem | Likely fix |
| --- | --- |
| Stepper vibrates but does not turn | Swap one coil pair; lower the speed; raise the A4988 current limit slightly |
| ESP32 resets when the pump starts | Add the 100 uF capacitor; separate the pump supply wires from the logic wires |
| ADS1115 not found | Check SDA/SCL, ADDR to GND, 3.3 V supply; run an I2C scanner |
| Glucose reading jumps around | Shorten and shield the sensor wires; add Rf capacitor; increase averaging |
| Web page cannot connect | Use the IP from the Serial Monitor; open the page locally (not via HTTPS); stay on the same Wi-Fi |
| Enzyme pump always runs | Gate pull-down resistor missing or MOSFET pins reversed |